A

PROJECT REPORT

ON

“Two Way RF Communication Module-WALKIE-TALKIE”

Submitted by

Tanmay Kamdi (251090003)

Aman Yadav (251090011)

Apurva Lambat (251091013)

Rehan Inamdar (251090029)

Prayas Meshram (251090043)

Om Shah (251090071)

F.Y. B.Tech.

UNDER THE GUIDANCE OF

Mr. Rohit M. Khadkikar

(Electrical Dept. VJTI, Matunga, Mumbai.)

VEERMATA JIJABAI TECHNOLOGICAL INSTITUTE, MATUNGA, MUMBAI.

Academic Year 2025-26

Semester-I

CERTIFICATE

This is to certify that, the Project report entitled

“Two Way RF Communication Module-WALKIE-TALKIE ”

Submitted by

Tanmay Kamdi (251090003)

Aman Yadav (251090011)

Apurva Lambat (251091013)

Rehan Inamdar (251090029)

Prayas Meshram (251090043)

Om Shah (251090071)

As the partial fulfillment of the Department Specific Workshop (DSW)

For the academic year 2025-26, Sem-I

This project is a record of student’s own work, carried out by them under our supervision and guidance.

Dr. Rajesh A. Patil							Mr. Rohit M. Khadkikar

ACKNOWLEDGEMENT

For all the efforts behind the project work, we first & foremost would like to express our sincere appreciation to the staff of the Department Specific Workshop (DSW), for their extended help & suggestions at every stage of this project.

It is with a great sense of gratitude that we acknowledge the support, time to time suggestions is highly indebted to our guide.

Finally, we pay our sincere thanks to all those who indirectly and directly helped us towards the successful completion of this project report.

GANTT CHART

|  | October | November |  |  |  | December |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Task | Week 1 | Week 2 | Week 3 | Week 4 | Week 5 | Week 6 | Week 7 |
| Finalize requirements & project objectives |  |  |  |  |  |  |  |
| background concept study |  |  |  |  |  |  |  |
| Schematic design on KiCad |  |  |  |  |  |  |  |
| component selection and buying |  |  |  |  |  |  |  |
| PCB/breadboard layout planning |  |  |  |  |  |  |  |
| Hardware assembly |  |  |  |  |  |  |  |
| Unit-level testing of each block |  |  |  |  |  |  |  |
| troubleshooting & design refinement |  |  |  |  |  |  |  |
| report/ppt formatting |  |  |  |  |  |  |  |
| Preparation of presentation |  |  |  |  |  |  |  |

PUGH CHART

| Design criteria | weight | Datum: 433 MHz PWM analog | 433 MHz FSK(frequency shift keying) | Use SOC Module (ESP32) |
| --- | --- | --- | --- | --- |
| Performance / range | 2 | . | + | 0 |
| Audio quality | 2 | D | ++ | ++ |
| Power consumption | 2 | A | - | -- |
| Cost | 2 | T | - | -- |
| Ease of implementation | 2 | U | - | -- |
| Reliability | 2 | M | + | 0 |
| Size/portability | 1 | . | 0 | + |
| Future scalability | 1 | . | + | ++ |
|  | + | 0 | 9 | 7 |
|  | 0 | 18 | 1 | 4 |
|  | - | 0 | 6 | 12 |
|  | TOTAL | 0 | 3 | -5 |

PAIR WISE COMPARISON CHART

|  | Functionality | Complexity | Cost | Power consumption | Maintainability | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| Functionality | ----- | 1 | 1 | 1 | 1 | 4 |
| Complexity | 0 | ------- | 0 | 0 | 0 | 0 |
| Cost | 0 | 1 | ----- | 0 | 0 | 1 |
| Power consumption | 0 | 1 | 1 | ------- | 1 | 3 |
| Maintainability | 0 | 1 | 0 | 0 | ------- | 1 |

| Sr. No. | Contents | Page No. |
| --- | --- | --- |
| 1 | Introduction | 7 |
|  | 1.1 Overview | 7 |
|  | 1.2 Objectives and Scope 1.2.1 Primary Objectives 1.2.2 Scope | 7 7 7 |
| 2 | Literature Survey | 8 |
|  | 2.1 Half-Duplex Communication system 2.2 RF Modulation/Frequency Selection | 8 8 |
| 3 | Block Diagram and Architecture | 9 |
|  | 3.1 System-Level Block Diagram | 9 |
|  | 3.2 Signal Flow and Operational Modes | 10 |
| 4 | System Design | 10 |
|  | 4.1 Power Supply and Regulation Circuit | 10 |
|  | 4.2 Transmitter Audio Conditioning chain 4.3 RF Transmission and Modulation 4.4 RF Reception and Demodulation 4.5 Receiver Audio Reconstruction 4.6 Push-to-Talk Control Logic | 11 12 12 13 14 |
| 5 6 7 8 9 | Component Specifications Circuit Diagram using KiCad Applications Conclusion, Future Scope and Enhancements References | 15 16 18 19 20 |

Chapter 1

Introduction

1.1 Overview

A compact walkie talkie transceiver has been developed to realize short-range, half-duplex wireless voice communication utilizing low-cost radio frequency hardware and discrete analog signal processing circuitry. The system employs a 9 V battery with on-board linear voltage regulation to provide a stable 5 V supply rail, followed by dedicated analog audio conditioning, pulse-width-modulation, and RF transmission stages that encode the microphone signal and drive a 433 MHz radio-frequency transmitter module.

On the receiver side, the demodulated PWM waveform is low-pass filtered to recover the baseband audio, which is subsequently amplified to loudspeaker level, while a push-to-talk control configures the unit to operate alternately in transmit or receive mode.

The project demonstrates fundamental principles of analog signal processing, modulation techniques, RF communication architecture, and practical circuit design methodologies suitable for undergraduate laboratory instruction and technical project development.

1.2 Objectives and Scope

1.2.1.  Primary Objectives:

- Design and fabricate a functional two-way wireless voice communication system operating at 433 MHz ISM (Industrial, Scientific, and Medical) band.

- Implement analog signal conditioning, modulation, and demodulation stages using discrete components and commercially available RF modules.

- Achieve transparent audio quality with minimal latency over wireless transmission distances up to 100–200 meters in open-air propagation.

- Provide hands-on experience with circuit design, PCB layout, RF integration, and system-level troubleshooting.

1.2.2. Scope:

- Two independent transceiver units, each capable of both transmission and reception.

- Half-duplex operation with manual push-to-talk switching.

- Frequency: 433 MHz ± 5 MHz (ISM band)

- Transmission range: 50–200 m.

- Audio bandwidth: ~300 Hz to 3 kHz (typical for voice communication)

- Power consumption: ~200–300 mW during transmission, ~50 mW in standby.

- Breadboard implementation for low cost and ease of assembly

Chapter 2

Literature Survey

2.1 Half-Duplex Communication Systems

- Half-duplex communication represents a mode of transmission in which data or information may be transmitted in both directions over a single communication channel, but not simultaneously.

- In contrast to full-duplex systems (which require independent transmit and receive paths), half-duplex systems are more economical, requiring only one RF transceiver stage per unit.

- Push-to-talk (PTT) switching is employed to arbitrate between transmit and receive modes, ensuring that the RF amplifier stages and audio output circuits operate without mutual interference.

- This architecture is prevalent in two-way radio systems including amateur radio (ham radio), commercial dispatch radio, and walkie talkie devices.

2.2 RF Modulation and Frequency Selection

2.2.1 Modulation Technique:

- The transmitted signal employs pulse-width modulation (PWM) as the carrier modulation scheme.

- The 433 MHz RF transmitter module internally generates a carrier waveform at the fundamental frequency, and the pulse-width-modulated signal gates this carrier on and off in accordance with the baseband audio envelope.

- This approach, termed on-off keying (OOK) or amplitude-shift keying (ASK) at the binary level, provides robust transmission over the ISM band with minimal spectral emissions outside the allocated bandwidth.

2.2.2 Frequency Selection:

- The 433 MHz frequency band is designated internationally as an ISM band, permitting unlicensed operation for short-range, low-power devices. This frequency offers a compromise between:

- Propagation characteristics: At UHF (300 MHz–3 GHz), wavelengths are approximately 70 cm, enabling practical antenna designs with quarter-wavelength elements ~17.3 cm long.

- Component availability: Miniaturized RF modules, antennas, and filter components are mass-produced and widely available at low cost.

- Regulatory compliance: Operation within ISM band allocations avoids frequency licensing and interference concerns.

Chapter 3

Block Diagram and Architecture

3.1 System-Level Block Diagram:

WIRELESS AUDIO COMMUNICATION SYSTEM

POWER SUPPLY BLOCK

• 9V Battery Input

• Voltage Regulator

• 5V Output

• Passive Filtering

TRANSMITTER AUDIO CHAIN

• Microphone Input

• Preamplification

• PWM Modulation

• Modulated Signal Output

RF TRANSMISSION BLOCK

• PWM Signal Input

• 433 MHz Carrier  Modulation (ASK/OOK)

• Antenna Radiation

RECEIVER AUDIO CHAIN

• Low-Pass Filtering

• Audio Signal Conditioning

•Audio Amplification and               speaker output

• Speaker Output

PTT CONTROL LOGIC (Transmit/Receive Arbiter)

• PTT (Push-To-Talk) Input

• Mode Switching Logic

• Transmitter Enable/Disable

• Receiver Enable/Disable

• Half-Duplex Operation Control

RF RECEPTION BLOCK

• Antenna Reception

• 433 MHz Signal Capture

• Demodulation to Baseband

• PWM Signal Recovery

3.2 Signal Flow and Operational Modes

Transmit Mode (PTT Pressed)

- Microphone → electret transducer generates ~10 mV AC signal

- MCP602 preamplifier boosts signal to ~500 mV

- TLC555 PWM modulator converts analog audio to ~40 kHz carrier with audio modulated duty cycle

- TC4427 MOSFET driver buffers and sharpens PWM edges

- 433 MHz TX module gates RF carrier in accordance with PWM envelope.

- Antenna radiates RF energy.

Receive Mode (PTT Released):

- Antenna intercepts RF signal from distant transmitter

- 433 MHz RX module demodulates RF to recover baseband PWM.

- LC low-pass filter (L₁, C₁₀) attenuates ~40 kHz switching frequency, recovering ~3 kHz audio bandwidth.

- LM386 audio amplifier boosts filtered audio to ~300 mW for speaker drive.

- Speaker converts electrical signal to acoustic output.

Chapter 4

System Design

4.1 Power Supply and Regulation Circuit

- Topology:

A linear voltage regulator (LM7805) reduces the 9 V alkaline battery voltage to a stable 5 V rail, powering all integrated circuits and RF modules.

Components:

- LM7805: 5 V linear regulator

    - Input (Pin 1): Unregulated battery voltage (7.5–12 V typical)

    - Ground (Pin 2): Common ground reference

    - Output (Pin 3): Regulated +5 V

- 470 µF: Large-value electrolytic capacitor at regulator input

    - Smooths battery voltage transients

    - Reduces input impedance for high-frequency disturbances

- 100 nF: Small-value ceramic capacitor at regulator output.

    - Decouples high-frequency switching noise.

    - Improves transient response of regulation loop

- Design Rationale:

Battery voltage drops as the cell discharges (9 V nominal, degrading to ~7.5 V at end of life). The LM7805 maintains constant 5 V output within this input range, ensuring stable supply to audio amplifiers and RF circuits. Ripple voltage at output is typically <50 mV peak, adequate for audio quality and RF stability.

4.2 Transmitter Audio Conditioning Chain

  - Microphone Preamplifier Stage:

Components:

  - Electret Microphone: Capacitive acoustic transducer yielding ~10 mV peak output at nominal voice levels.

  - MCP602: Dual operational amplifier

- Low input offset voltage (~1 mV typical)

- Rail-to-rail input and output capability

- Low quiescent current (~1 mA per channel) suitable for battery operation

4.3 RF Transmission and Modulation

- PWM Modulation Stage:

Components:

- TLC555:  CMOS 555 timer in a stable multivibrator configuration.

- Operating Principle:

- The TLC555 functions as a voltage-controlled oscillator (VCO) wherein the preamplified audio signal modulates the control voltage at the threshold/trigger input .

- As the instantaneous audio voltage fluctuates, the internal capacitor charge/discharge timing shifts, causing the output duty cycle to vary in proportion to the audio amplitude.

- The output is a pulse-width-modulated (PWM) square wave at fixed frequency with pulse width varying between ~20% and ~80% in response to voice modulation.

- MOSFET Driver Stage:

Components:

- TC4427: High-current MOSFET driver

- Peak output current: ±1.5 A

- Propagation delay: ~15 ns

- Input thresholds: logic-level compatible with CMOS output

- Supply voltage: 5 V

Function:

The TLC555 output, while logic-compatible, cannot source sufficient current to reliably drive the 433 MHz RF module input (typical input impedance ~50 Ω, requiring ~100 mA peak). The TC4427 accepts the 555 output and delivers high-current, fast-edge PWM pulses to the RF module data input.

4.4 RF Reception and Demodulation

- 433 MHz Receiver Module:

- Components:

- RF RX Module: ASK/OOK demodulator integrated circuit

- Frequency: 433 MHz ±5 MHz

- Sensitivity: ~-100 dBm at 2400 baud

- Output: TTL-logic-level PWM envelope

- Demodulation Process:

The receiver module contains:

- RF front-end: LC matching network, low-noise amplifier (LNA), limiting amplifier

- Intermediate-frequency (IF) stage: Typically, 1–3 MHz, with narrow-band filter

- Envelope detector: Extracts modulation envelope from IF carrier, outputting binary logic levels.

The demodulated PWM at the data output pin contains the same pulse width variations as the transmitted signal, having traversed the RF path with negligible distortion (provided signal-to-noise ratio remains >10 dB).

4.5 Receiver Audio Reconstruction

- Low-Pass Filter Stage:

Components:

- 1.5 mH: Series inductor.

- 10 µF: Shunt capacitor to ground

- optional, 1 kΩ: Series resistance for damping

Filter Design:

The LC low-pass filter attenuates the ~40 kHz PWM carrier while preserving the ~3 kHz  audio baseband. The cutoff frequency is approximately:

fc=12πLC=12π1.5*10-3*10*10-6 =1.3 kHz

This passive filter provides ~40 dB attenuation at 40 kHz (12 dB/octave Rolloff ), effectively removing switching noise while passing voice frequencies. The filtered output is a reconstructed analog waveform approximating the original microphone signal (~50–500 mV amplitude, depending on transmit input level).

- Audio Amplification Stage:

Components:

- LM386: Audio power amplifier, 1 W continuous into 8 Ω

- Input impedance: ~50 kΩ (Pin 3 non-inverting input)

- Output impedance: ~75 Ω

Configuration:

- Pin 3 (Inverting Input): Grounded through coupling capacitor to AC-pass low frequency components

- Pin 2 (Non-Inverting Input): Receives filtered audio signal through coupling capacitor

- Pin 6 (Positive Supply): +5 V

- Pin 4 (Ground): Signal ground

- Pins 1–8 (Gain-Setting Capacitor): 10 µF capacitor sets gain to ~200× (46 dB)

- Pin 5 (Output): Coupled to speaker through 220 µF capacitor (DC blocking)

Output Power:

With 5 V supply and 8 Ω load, maximum output power is approximately:

pmax=vcc22RL=2.528= 0.78W

In practice, delivering ~300–500 mW to an 8 Ω loudspeaker produces sufficient acoustic output (85–90 dB SPL at 1 m) for intelligible speech in typical indoor/outdoor environments.

4.6 Push-to-Talk Control Logic

Functionality-

The push-to-talk switch implements half-duplex arbitration between transmitter and receiver paths.

Power Control- The PTT switch directly gates power to the transmitter chain (TLC555, TC4428, TX module). When PTT is pressed, transmitter power is enabled; when released, transmitter powers down and receiver is active. This approach minimizes idle-state power consumption.

Chapter 5

Components specifications

Table 1: Active Components

| COMPONENTS | QUANTITY | FUNCTION |
| --- | --- | --- |
| LM7805 (5 V Regulator, TO-220) | 2 | Power supply |
| MCP602 (Dual Op-Amp, DIP-8) | 2 | Mic preamplification |
| TLC555 (CMOS 555 Timer, DIP-8) | 2 | PWM modulator |
| TC4427 (MOSFET Driver) | 2 | TX driver |
| LM386 (Audio Power Amp, DIP-8) | 1 | Speaker driver |
| 433 MHz TX/RX Module Pair | 2 | RF transceiver pair |
| Electret Microphone Capsule | 2 | Audio input |
| 8 Ω Speaker (0.5–1 W, 40 mm) | 2 | Audio output |

Table 2: Passive Components

| COMPONENTS | QUANTITY | FUNCTION |
| --- | --- | --- |
| Resistors (1/4 W, mixed values) | 40pcs | DC biasing, feedback gain setting, LC filter damping, etc. |
| Ceramic Capacitors (10 nF 100 nF) | 20pcs | Decoupling, filtering |
| Electrolytic Capacitors (10 µF–470 µF) | 16pcs | Filtering, coupling |
| Inductors (1.5 mH) | 2 | LC filter |
| Potentiometers (10 kΩ with knobs) | 2 | Volume control |
| Push-buttons (PTT switches) | 2 | Manual transmit control |
| Power Switches | 2 | Unit on/off |
| Antenna Wire (17.3 cm, from 1 m roll) | 2 | Radiat-ing element |

Table 3: Hardware and Enclosure

| ITEM | QUANTITY |
| --- | --- |
| Perfboard (7×9 cm or similar) | 2 |
| 2-pin Battery Connectors | 2 |
| Header Pins and Connectors | 10sets |
| 9 V Battery Clip | 2 |
| Hookup Wire, Solder, Heat Shrink Screws, Standoffs, Misc. Hardware | shared |

Table 4: Power and Enclosure

| ITEM | QUANTITY |
| --- | --- |
| Plastic Enclosures (ABS/Polycarbonate) | 2 |
| 9 V Alkaline Batteries | 2 |

Chapter 6

Circuit Diagram using KiCad

Chapter 7

Applications

The 433 MHz walkie-talkie transceiver demonstrates direct applicability to numerous practical domains:

1. Short-Range Personal Communication: Event coordination, outdoor recreation, camping, hiking

2. Educational Demonstration: Undergraduate laboratory for analog circuit design, RF communication, signal processing

3. Industrial Dispatch: Warehouse inventory management, construction site coordination, factory floor communication

4. Emergency Response: Backup communication during cellular outages, disaster recovery scenarios

5. Hobby Electronics: Amateur radio experimentation, robotics projects requiring wireless control

6. IoT Sensor Networks: Low-power telemetry and data logging in remote locations with licensing constraints

7. Home Automation: Wireless control of appliances, door locks, lighting without dependency on cellular infrastructure

Chapter 8

Conclusion, Future Scope and Enhancements

The design and implementation of a 433 MHz walkie-talkie transceiver successfully demonstrate the integration of analog signal processing, digital modulation, and RF transmission/reception techniques into a functional, low-cost communication system.

The circuit employs discrete components and commercially available RF modules to achieve transparent voice communication over distances of 50–200 meters, suitable for short-range personal and industrial use.

- Hardware Enhancements:

- Frequency Agility: Replace fixed-frequency RF modules with tenable VCO-based transmitters/receivers to support multiple channels within the ISM band.

2. Antenna Design: Implement directional (Yagi or patch) antennas to increase range and reduce interference

3. Audio Filtering: Add higher-order active filters (e.g., Sallen-Key topology) to improve voice clarity and reduce adjacent-channel interference

4. Power Amplification: Integrate a class-C RF power amplifier to increase transmit power (within regulatory limits) and extend range to 1+ km

5. Digital Signal Processing: Employ a microcontroller or DSP to implement advanced modulation schemes (e.g., FSK, GFSK) for improved spectral efficiency.

- Functional Enhancements:

1. Full-Duplex Operation: Design separate transmits and receive RF paths with duplexing filters to enable simultaneous two-way communication

2. Channel Memory: Store frequently-used frequencies in firmware for rapid selection

3. Signal Strength Indication (RSSI): Display received signal strength via LED bar graph or LCD panel

4. Battery Management: Implement low-battery warning and automatic shutdown to extend cell life

5. Encryption: Add digital encryption (e.g., AES-128) for secure communication in sensitive applications

Chapter 9

References

1)Texas Instruments. (2024). TLC555 CMOS Timer—Datasheet. https://www.ti.com/product/TLC555

2) Microchip Technology. (2024). MCP602 Dual Op-Amp—Datasheet. https://www.microchip.com/en-us/product/MCP602

3) Intersil. (2024). LM386 Audio Power Amplifier—Datasheet. https://www.intersil.com/products/power-management/lm386

4) Texas Instruments. (2024). LM7805 5 V Regulator—Datasheet. https://www.ti.com/product/LM7805